1. List 5 common hardware issues you might encounter in a desktop PC and write the typical symptom for each (for example: 'No display on monitor' — symptom: blank screen, no POST beep).

Ans :-

| No. | Hardware issue | Typical symptom |
| --- | --- | --- |
| 1 | RAM failure | PC does not boot or gives beep sounds. |
| 2 | Hard disk/SSD failure | Computer becomes slow or cannot load the operating system. |
| 3 | Monitor or display problem | Blank screen or no display. |
| 4 | Power supply (SMPS) failure | PC does not turn on or shuts down suddenly. |
| 5 | CPU overheating | PC becomes hot, slows down, or shuts down automatically. |

2. Given this scenario: A laptop is not charging even when plugged in. Write down a step-by-step troubleshooting plan using only basic tools (no software diagnostics), and explain what you would check at each step.

Ans :-    Check the power socket: Plug another device into the socket to confirm it works.

  Check the charger: Inspect the adapter and cable for cuts, damage, or loose connections.

  Check the charging port: Look for dust, damage, or a loose connection. Do not insert metal objects.

  Check the charging indicator: Connect the charger and see whether the laptop's charging light turns on.

  Try another compatible charger: Use a known-working charger with the correct voltage and suitable power rating.

  Check the battery: If the battery is removable, disconnect the charger, remove the battery, and inspect it for damage. Never use a swollen battery.

  Check the result: If the laptop still does not charge, the battery, charging port, or motherboard charging circuit may be faulty. Take it to a technician.

3.  Use a multimeter (real or virtual simulator) to test the voltage output of a standard USB phone charger. Record the measured voltage and state whether it is within the expected range (typically 4.75V to 5.25V for USB).

Ans :-  USB phone charger, USB breakout adapter, and digital multimeter.

Steps:

- Set the multimeter to DC voltage (V⎓).

- Connect the charger to a working power outlet and use a USB breakout adapter to access the output terminals safely.

- Place the red probe on the USB +5V (VBUS) terminal and the black probe on GND.

- Read and record the voltage shown on the multimeter.

Example :

| Item | Result |
| --- | --- |
| Expected voltage | 4.75 V to 5.25 V |
| Measured voltage (example) | 5.02 V |
| Conclusion | Within the expected range; charger output is normal. |

4. A PC is randomly shutting down. Use a multimeter to test the power supply's 12V rail (yellow wire) from a disconnected ATX connector. Describe the steps and write down the voltage you find. If the voltage is below 11.5V, what hardware issue could this indicate?<br><br><em><strong>Hint:</strong> Remember to set the multimeter to DC voltage and probe between the yellow and black wires.</em>

Ans :-   Steps:

- Turn off the PC and unplug the power supply from the wall.

- Disconnect the ATX power connector from the motherboard.

- Set the multimeter to DC voltage (V⎓).

- Identify the yellow wire (+12 V) and a black wire (ground) on the disconnected connector.

- With the power supply unplugged, do not attempt to measure its live output. A proper PSU tester or a supervised, safely configured test setup is needed to power it for measurement. Never open the PSU casing.

- Record the voltage only when measured using a safe, appropriate test setup.

Example :

| Item | Result |
| --- | --- |
| Expected 12V rail | Approximately 12 V |
| Example measured voltage | 11.3 V |
| Conclusion | Below 11.5 V; the power supply may be faulty or unable to maintain the required voltage. |

- A voltage below 11.5 V may indicate a failing or overloaded power supply (SMPS). This can cause random shutdowns, system instability, or unexpected restarts. Other hardware faults may also cause these symptoms, so further testing is needed.