1.  A user reports their laptop won't power on even when plugged in. List 5 troubleshooting steps you would take to diagnose the cause, and for each step, explain what you are checking for.

Ans :-    Check the power socket: Plug another device into the socket to confirm that it is working.

  Check the charger: Inspect the charger and cable for damage, and check whether the charging light turns on.

  Check the battery: If the battery is removable, disconnect it and try starting the laptop with the compatible charger only, if supported.

  Perform a power reset: Unplug the charger and disconnect removable accessories. Follow the manufacturer's power-reset instructions, then reconnect the charger and try turning on the laptop.

  Check for hardware problems: Look for charging lights, fan noise, or other signs of power. If the laptop still does not start, the battery, charging port, or motherboard may be faulty.

2. You receive a helpdesk ticket: 'My monitor is showing a blank screen, but the CPU seems to be running.' Write a step-by-step troubleshooting flow you would follow to identify and resolve the issue.<br><br><em><strong>Hint:</strong> Consider checking both hardware connections and basic display settings.</em>

Ans :-    Check the monitor power: Make sure the monitor is switched on and its power cable is connected properly.

  Check the display cable: Ensure the HDMI, DisplayPort, or VGA cable is firmly connected to the monitor and computer.

  Check the monitor input: Select the correct input source, such as HDMI or DisplayPort.

  Check the brightness: Increase the monitor brightness and check whether the screen displays anything.

  Check the computer: Restart the computer and observe whether the startup logo appears.

  Test another cable or monitor: Use a working cable or another monitor to identify whether the original monitor or cable is faulty.

  Check display settings: If the computer starts but the screen remains blank, check the display mode and graphics driver when possible. On Windows, press Windows + P to check the display mode.

- These steps help identify whether the problem is caused by the monitor, cable, display settings, or computer hardware.

3.  A user complains that their wireless mouse is unresponsive, but the keyboard works fine. Describe how you would isolate whether the issue is with the mouse, the USB port, or the system settings.

Ans :-    Check the mouse battery: Replace or recharge the battery and ensure the mouse is switched on.

  Check the USB receiver: If the mouse uses a USB receiver, remove it and reconnect it securely.

  Try another USB port: Connect the receiver to a different USB port to check whether the original port is faulty.

  Test the mouse on another computer: If it still does not work, the mouse or receiver may be faulty.

  Check system settings: Open Windows Device Manager and check the mouse and related USB devices for errors. Update or reinstall the relevant driver if needed.

- If the mouse works on another computer, check the original computer's USB port, driver, and settings. If it fails on multiple computers, the mouse or receiver may be defective.

4.  Imagine you are working on a helpdesk team for a gaming cafe. One system is overheating and shutting down during use. List 3 possible hardware causes and describe how you would check each one.<br><br><em><strong>Hint:</strong> Think about fans, dust, and thermal paste.</em>

Ans :-

| Hardware cause | How to check |
| --- | --- |
| 1. Faulty cooling fans | Check whether the CPU and graphics card fans spin properly during operation. Replace a faulty fan if necessary. |
| 2. Dust buildup | Inspect the fans, air vents, and heatsinks for dust. Turn off and unplug the PC before cleaning them carefully with suitable computer-cleaning equipment. |
| 3. Old or poorly applied thermal paste | Check whether the CPU is overheating even though its cooler is working. A qualified technician can inspect and replace the thermal paste if necessary. |

- Check the cooling fans, clean dust from the system, and inspect the CPU cooling system. These steps can help prevent overheating and unexpected shutdowns.

- Always shut down and unplug the computer before inspecting or cleaning internal components.